Aquilla 12  x  NP-C4I Recorder

Call Recording Integration - User Guide

Eagleies Creative

# 1.  What this does

Aquilla 12 now mirrors every peer-to-peer call into the NP-C4I Recorder automatically. When you are on a call, both directions of the conversation - your voice (transmit) and the other user's voice (receive) - are streamed to the recorder and saved to disk. No extra button-pushing during the call; you just talk.

Under the hood Aquilla sends a clean copy of the call audio as standard RTP/PCMU (G.711 u-law) to a UDP port that the recorder is already listening on. Both directions share one stream, so each call becomes one combined recording file containing the whole conversation. This works even when the live call itself is encrypted (SRTP) - the recorder always receives a clean, unencrypted copy.

## Channel to recorder port mapping (defaults)

Each Aquilla channel maps to one recorder channel/port. The defaults below already match the NP-C4I Recorder factory config.json:

| Aquilla channel | Recorder UDP port | Recorder channel name |
| --- | --- | --- |
| CH 01 | 5004 | Tactical Net Alpha |
| CH 02 | 5006 | Command Channel 1 |
| CH 03 | 5008 | Air Support Link |
| CH 04 | 5010 | Logistics Dispatch |

Note: out of the box the recorder listens on four ports, so Aquilla channels 1-4 are recorded. To record more channels, add matching channels/ports to the recorder config.json (see Section 5).

# 2.  One-time setup

1.  Point Aquilla at the recorder. On first run Aquilla creates a settings file called recorder_config.json. A copy also ships in the SIP Controller project folder for reference. Its contents:

{
  "enabled": true,
  "ip": "127.0.0.1",
  "base_port": 5004,
  "port_step": 2
}

Leave it as-is if the recorder runs on the same computer as Aquilla. If the recorder runs on a different machine, change "ip" to that machine's address.

| Field | Meaning | Default |
| --- | --- | --- |
| enabled | Master on/off switch for recording mirror | true |
| ip | Address of the machine running NP-C4I Recorder | 127.0.0.1 |
| base_port | UDP port used for Aquilla channel 1 | 5004 |
| port_step | Port gap between channels (keep at 2) | 2 |

Where the live file lives: macOS ~/Library/Application Support/Aquilla-12/recorder_config.json  .  Windows %APPDATA%\Aquilla-12\recorder_config.json. Edit that file (not the reference copy), then restart Aquilla.

# 3.  Recording a call (day-to-day)

1.  Start NP-C4I Recorder and leave it running. Confirm the channels show Idle or Streaming (not Disconnected).

2.  In the recorder, turn Recording ON. The recorder only writes files while its Recording toggle is enabled.

3.  Open Aquilla 12 and place or accept a peer-to-peer call as normal.

4.  Talk. Hold PTT to transmit; your outgoing audio is captured only while PTT is active. The other user's incoming audio is captured whenever they speak.

5.  Hang up when done. The recorder finalizes the audio file for that channel.

You can confirm it is working: the recorder channel row for that call flips to Streaming and its VU/volume meter moves while either party talks.

# 4.  Finding your recordings

Files are written by the recorder to its storage folder, chunked automatically (default every 15 minutes):

JPS_Recorder/NP4-C4I/recordings/

Each channel records to its own file, and every recording event is also logged in the recorder database (NP4-C4I/recorder.db). Chunk length and storage location are set in the recorder config.json (chunk_duration_minutes, storage_dir).

# 5.  Recording more than 4 channels

To record Aquilla channels 5 and up, add matching entries to the recorder config.json so it listens on the next ports. The port for any Aquilla channel is:

port = base_port + (channel_number - 1) x port_step
example: CH 05  ->  5004 + (5-1)x2  =  5012

So for CH 05 add a recorder channel on port 5012, CH 06 on 5014, and so on. Keep port_step the same (2) on both sides.

# 6.  Troubleshooting

| Symptom | Likely cause and fix |
| --- | --- |
| No file is created | Recording toggle is OFF in NP-C4I. Turn it ON. Also confirm enabled is true in recorder_config.json. |
| Recorder channel stays Idle | No audio yet - nobody has spoken, or PTT not held. It flips to Streaming on first voice. |
| Recording on wrong channel | Aquilla channel number and recorder port must line up (CH1->5004, CH2->5006 ...). Check base_port and port_step match on both apps. |
| Recorder on another PC gets nothing | Set ip in recorder_config.json to that PC address, and allow UDP ports 5004-5010 through its firewall. |
| Only one side of the call is recorded | You capture your own voice only while holding PTT. Hold PTT while speaking; the far side records automatically. |
| Settings change ignored | Edit the file in the Aquilla-12 app-data folder (not the project copy), then restart Aquilla. |

Aquilla 12 x NP-C4I Recorder - Eagleies Creative